Plant Pal — Smart Soil Watering System

Overview

Plant Pal is an embedded microcontroller system that monitors soil moisture, air temperature, and humidity to determine when a plant needs watering.
It operates through well-defined system states, uses interrupts, performs manual ADC sampling, keeps real-time event logs, and provides user feedback through LEDs, an LCD, and a small motor or pump.

Functional Requirements

Inputs & Sensors

| Component | Purpose | Implementation Notes |
| --- | --- | --- |
| Soil moisture sensor | Measures soil wetness via ADC | Use direct register-level ADC (no analogRead()) |
| DHT11(temperature Sesor) | Reads temperature & humidity | Library allowed |
| Start/Stop button | Enables or disables the system | Handled via ISR interrupt |
| Reset button | Clears error state | Polled input |

Outputs

| Component | Function |
| --- | --- |
| LCD Display | Shows soil status, temperature, and humidity |
| LED Indicators | Show states — Yellow for Disabled, green for Idle, blue for Running and red for Error |
| Pump or Fan | Waters the plant when soil is dry |
| RTC Module | Logs timestamps for state changes and events |
| Serial Port | Sends event messages (e.g., “Soil dry at 3:22 PM”) to a host computer |

System States

| State | Description | Behavior |
| --- | --- | --- |
| DISABLED | System off | Turn Yellow led on, no sensor activity; ISR monitors Start button. |
| IDLE | Monitoring normal conditions | Turn Green led on; reads sensors every minute; transitions to RUNNING if soil dry. |
| RUNNING | Watering active | Turn Blue led on; pump runs until soil returns to normal; logs activation time. |
| ERROR | Fault state | Turn Red led on, pump off, LCD shows “Sensor Error” or “Low Water”, reset button returns to IDLE if resolved. |

Timing and Event Handling

- Use the RTC for accurate timestamps and event logging.

- Use millis() for 1-minute intervals (no delay() calls).

- Start/Stop button handled via external interrupt (attachInterrupt() allowed).

- LCD updates once per minute during active states.

- All transitions are reported over Serial with timestamps.

Allowed Libraries

| Feature | Library Use |
| --- | --- |
| RTC module | Allowed |
| LCD display | Allowed |
| DHT11 sensor | Allowed |
| Motor/relay control | Allowed |
| GPIO, ADC, Serial functions | Use register-level code only |

Hardware List

- Arduino Mega 2560

- Soil moisture sensor (analog input)

- DHT11 temperature/humidity sensor

- RTC module (I²C)

- 16×2 LCD (I²C or parallel)

- 4 LEDs for state indication

- Push buttons (Start/Stop and Reset)

- Pump or small DC motor for watering control

System overview (what always runs)

- Clock & logging: The RTC is your time source; every state change and noteworthy event is timestamped and reported over Serial.

- Scheduler: One central “tick” based on millis() drives periodic actions (no delay()):

    - Sensor cadence: once per minute in active states.

    - LCD cadence: refresh once per minute (and immediately on state changes).

- Buttons:

    - Start/Stop: external interrupt; toggles between DISABLED ↔ IDLE, and can stop RUNNING/ERROR back to DISABLED.

    - Reset: polled; only affects ERROR (tries to recover to IDLE if the fault is gone).

- LEDs: Always indicate the current state: red, green, blue and yellow.

Power-up & initialization

- Boot & self-check (instant):

  - Initialize peripherals (ADC in manual mode, GPIO directions, LCD, DHT11, RTC, motor/relay control, Serial).

  - If any hard fault is detected (e.g., ADC not ready, RTC missing), enter ERROR; otherwise proceed.

- Enter DISABLED by default:

  - Turn Yellow on!

  - Actuators: pump OFF.

  - Sensors: no periodic sampling (conserve power/avoid noise).

  - ISR for Start/Stop is armed.

  - LCD: “System Disabled” + time.

  - Log: “System entered DISABLED @ ”.

Start/Stop interrupt behavior (any time)

- If in DISABLED and Start is pressed (ISR):

  - Timestamp and log “Start pressed”.

  - Transition → IDLE (see below).

- If in IDLE or RUNNING or ERROR and Stop is pressed (ISR):

  - Timestamp and log “Stop pressed”.

  - Force actuators safe (pump OFF), clear minute timers, transition → DISABLED.

IDLE (monitoring)

Goal: Watch the environment and be ready to water if soil is dry; lightweight behavior.

- Entry actions:

  - LEDs: Green led on.

  - Pump Off.

  - LCD immediate update: “IDLE” + last known readings or “waiting for next sample”.

  - Log: “Entered IDLE @ ” IDLE Time

- Minute tick (every 60s via millis() ):

  - Soil moisture sample (manual ADC):

    - Take multiple quick reads (e.g., 3–5) and average to reduce noise.

    - Validate reading range to catch open/short conditions.

  - DHT11 read:

    - Get temperature & humidity, validate checksum/status from the library.

  - LCD update:

    - Line 1: Soil status (e.g., “Soil: Dry/OK + raw%”).

    - Line 2: “T: {number} °C  H: yy%”.

  - Decision logic:

    - Sensor fault? (ADC out of plausible range, DHT error):

      - Transition → ERROR (“Sensor Error”); log fault type and values.

    - Soil dry? (below dry threshold with hysteresis and debounce):

      - Require “dry” for N consecutive samples (e.g., 2–3 minutes) to avoid false starts.

      - When confirmed, transition → RUNNING; log “Soil dry—watering started”.

    - Soil OK: remain in IDLE.

- While waiting: no blocking—system remains responsive to the Start/Stop ISR.

RUNNING (watering)

Goal: Water until soil returns to acceptable moisture to protect plant and hardware.

- Entry actions:

  - LEDs: Blue led on.

  - Pump ON (or open relay).

  - Record watering session start time: log event with timestamp.

  - LCD immediate update: “RUNNING: Watering…”.

- Safety rails active while RUNNING:

  - Max continuous run time (e.g., 2–5 minutes default): if exceeded → ERROR (“Timeout / Low Water?”).

  - Thermal/overcurrent hooks (optional if hardware supports): if tripped → ERROR.

  - Minute tick loop: same cadence as IDLE.

- Minute tick (every ~60s):

  - Soil re-sample (manual ADC): average & validate.

  - DHT11 read (for logging/LCD).

  - LCD update: show “Watering t=mm:ss”, current soil %, T/H.

  - Decision logic:

    - Sensor fault? → ERROR (“Sensor Error”), pump OFF, log.

    - Soil recovered? (≥ wet threshold using hysteresis):

      - Confirm with 1–2 consecutive OK samples to prevent chattering.

      - Then stop pump, log “Watering stopped; soil OK”.

      - Transition → IDLE.

    - Still dry: continue; keep monitoring max-time limit.

- Exit actions (on any transition): pump OFF, log stop reason & duration.

ERROR (fault handling)

Goal: Fail safe, inform the user, and recover only when the problem is gone.

- Entry actions:

  - Turn LED Red on

  - Pump OFF (force safe).

  - Freeze any watering timers.

  - Log specific fault (“Sensor Error”, “Low Water”, “Pump Timeout”, etc.) with raw readings.

  - LCD: big, clear message (e.g., line 1: “ERROR”; line 2: reason).

- While in ERROR:

  - No watering; Start/Stop still works to go to DISABLED.

  - The system continues minute-tick diagnostics to see if conditions normalize (so the Reset button can succeed). This includes:

    - Re-reading soil and DHT11.

    - Optional: checking a reservoir/float switch if present (for “Low Water” case).

  - Reset button (polled):

    - When pressed:

      - If the original fault condition is no longer present, log “Reset acknowledged—fault cleared”, then transition → IDLE.

      - If the fault persists, stay in ERROR; update LCD (“Still in Error”) and log the failed reset attempt with current readings.

LCD update policy (clarity without flicker)

- Immediate updates on every state change.

- Periodic updates once per minute in IDLE/RUNNING/ERROR.

- Keep messages compact and consistent so a user can read state + last sample at a glance.

Debounce, hysteresis, and stability (avoid false actions)

- Start/Stop button: hardware/ISR debounce (time-based ignore window or edge validation).

- Soil dryness:

- Hysteresis: Two thresholds (e.g., Dry < 35%, Wet > 45%).

- Debounce: require N consecutive dry/ok confirmations before switching states.

- Sensor sanity checks:

  - ADC plausible range; reject saturated readings as faults.

  - DHT11 checksum/status must pass; otherwise fault.

Logging & timestamps (what you report)

- State changes: DISABLED↔IDLE, IDLE→RUNNING, RUNNING→IDLE, any → ERROR, ERROR→IDLE (after reset).

- Watering sessions: start time, stop time, duration, average soil level during session.

- Faults & recoveries: type, raw values, time.

- Periodic notes (optional): brief heartbeat once every few minutes (“IDLE heartbeat T/H & soil%”) to help debugging.

Edge cases & priorities

- Start/Stop during RUNNING: immediately safe-stop pump, go to DISABLED, log user abort.

- Power loss mid-run: on reboot, you start in DISABLED; previous session can’t be resumed—log “Unexpected restart”.

- Repeated faults: if the same error occurs X times in Y minutes, escalate the LCD message (“Check wiring/sensor”) but remain in ERROR until Reset + recovery.

- Empty reservoir (if applicable): treat like a dedicated Low Water fault (go to ERROR); only recover after refill + Reset.